The following items should be checked on the trailer and tow vehicle every time you tow a trailer, even for short trips. You may want to copy this list and keep it in the tow vehicle.

On the tow vehicle, check:

- Fluid levels:  fuel, oil, transmission fluid, brake fluid, and radiator

- Tire pressure: examine tires for cracks, bulges or signs of excessive wear

- Hitch and receiver: no rust, loose bolts, or cracked welds; ball is correct size for trailer and is tight

- Electrical connectors and wiring on both vehicles (no loose connectors or broken wires).

- With the trailer hitched, check running lights, turn indicators, brake lights and mirror adjustment

- If applicable, with the trailer hitched, test the brakes and trailer brake controller, and adjust trailer brakes for the load

On the trailer, check:

- Hitch and coupling, be sure coupler fastens securely over ball

- Check safety chains, hooks, and points where they connect

- Tire pressure and condition of all tires

- Signal and brake lights, running lights, and emergency flashers

- Check that the pin securing the ball mount to the receiver is intact

- Check that load in trailer is secure so that no loose objects will bounce or fly out

- The trailer is licenced and I have a copy of the ownership

Towing Considerations

- Reduced braking capacity

- Increased fuel consumption

- Reduced safe passing opportunities

- Reduced rear vision

- Reduced acceleration

- Increased driver fatigue

- Increased wind effects

- Height of trailer when reversing (car ports, overhanging trees)

- Propensity to jack knife when reversing